CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Francisco Mata Carlos

Student #: 1012593607

Student Email: matacarl@unlv.nevada.edu

Primary Github address: https://github.com/chicosisco/da_sub.git

Directory: repository/cpe301/DesignAssignments/DA3A

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

The components used for this assignment are the next:

- Atmega328p Xplained Mini

- Atmel Studio 7

- FTDI chip

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Task 1_A

1.  Write a C AVR program that will display a string, random integer and floating point values on the serial terminal every 1 sec. Use a timer with interrupt for the 1 sec delay. Use a FTDI chip for serial to USB conversion

/*

* DA3A.c

*

* Created: 3/28/2019 2:24:27 AM

* Author : Francisco Mata carlos

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdio.h>

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL)))-1)

//Function declarations

volatile int Count;

void USART_init( unsigned int ubrr );

void USART_TX_string(char *data);

char outs[30];

int x_num;				      //creates variable for random number

float due_date;			      //creates variable for decimal point

char str[] = "July_4th_1776";		//creates string

char empty[] = " ";			      //creates space

int main(void)

{

Count = 0;

TIMSK0 |= (1<<TOIE0);		//sets interrupt when overflow occurs

sei ();			      //set global overflow

TCCR0A = 0;										//normal mode

TCCR0B |= (1<<CS02)|(1<<CS00);					//prescaler = 1024

USART_init(BAUD_PRESCALLER);					//baud prescaler

USART_TX_string("connection_successful\r\n"); //prints the word connection successful when connected and running

while (1);

}

// USART (RS-232)

void USART_init( unsigned int ubrr ){

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0);

UCSR0C = (3 << UCSZ00);

}

//sending string to TEH RS-232

void USART_TX_string(char *data) {

while (*data != '\0') {

while (!(UCSR0A & (1<<UDRE0)));

UDR0 = *data;

data++;

}

}

ISR (TIMER0_OVF_vect){			//timer0 overflow interrupt call

while (Count < 61){

if ((TIFR0 & 0x01) == 1){	//checks for overflow flag

TIFR0 = 0X01;	       //reset overflow

Count++;

}

}

if (Count > 60){

USART_TX_string(str);	//prints string

USART_TX_string(empty);	//prints space

x_num = rand();		// random number

due_date = 327.19;		//creates floating value; due date of assignment

snprintf(outs, sizeof(outs), "%3d\r\n", x_num);

USART_TX_string(outs);

USART_TX_string(empty);

sprintf(outs, "%f", due_date);

USART_TX_string(outs);

USART_TX_string(empty);

Count = 0;

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Same as above

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Task 1_A  C code

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task 1

Photo below shows the set up

- VIDEO LINKS OF EACH DEMO

DA3A

https://youtu.be/vQLKcaiOqFk

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Francisco Mata Carlos